Brewmaster: Overall responsibility for the brewing process, rest times, calculating malt and hop additions.

Brewer: Responsible for weighing, milling, measuring, and sanitising tasks. Ensures all measurements and processes meet quality standards.

Notes:

First brew = must be selected / confirmed for the first batch of the day. Time must be entered correctly for mash in.

Last brew = Must be selected / confirmed for the last batch of every day.

Checklist Day Before Brewing

- CIP FST tank – confirm with cellar/brewer

- 2 big bags attached and full (confirm on HMI)

- Weigh malt for first batch and confirm hopper on HMI

- Mill Batch 1 (first brew button and confirm mash in date and time)

- Weigh malt for following batch and confirm hopper on HMI

- Check IBC Container

Brew Day

| For First Batch: | For following batches: | For last batch of tank |
| --- | --- | --- |
| - Requires button “first brew” & mash in time - Measure First Wort Extract - Mill next batch - Attach hose and t-piece to bottom inlet of tank - Start cleaning wort way & pitching line - Measure last running - Weigh hops and place in doser (confirm on HMI) - Measure kettle full extract - Measure cast out extract & PH - Pitch yeast into batch according to cell count - Aeration 350 kg/m3 - End cooling | - Measure First Wort Extract - Mill next batch - Attach hose and t-piece to top inlet of tank - Start sterilise wort way - Measure last running - Check IBC container - Weigh hops and place in doser (confirm on HMI) - Measure kettle full extract - Measure cast out extract & PH - Aeration 350 kg/m3 - End cooling | - Measure First Wort Extract - Start sterilise wort way - Measure last running - Check IBC container - Weigh hops and place in doser (confirm on HMI) - Measure kettle full extract - Measure cast out extract & PH - No Air - End cooling |

Quality Control

Ensure all measurements are within the specified range.

Record all data accurately in the brewing log.

Address any deviations immediately to maintain the quality of the brew.

Health and Safety

Wear appropriate PPE (Personal Protective Equipment) at all times.

Follow all safety guidelines for handling chemicals and equipment.

Report any accidents or safety concerns to the Brewmaster immediately.

Documentation

Maintain a detailed log of all weights, measurements, and actions taken during the brewing process.

Document any issues or deviations and the corrective actions taken.